Appendix 1 - Definitions

This section will define terms in accordance with Air Force Instruction (AFI) 32-1132, Airport and Helipad Planning and Design.

Runway: A defined rectangular area of an airfield prepared for fixed-winged aircraft use during aircraft launch and recovery operations.  Runways are classified as either Class A or Class B based on aircraft types as shown in Table 1.

| Class A Runways | Class B Runways |
| --- | --- |
| C-1, C-2, C-12, C-20, C-21, C-23, C-26, E-1, E-2, OV-1, T-3A, T-28, T-34, T-44, U-21, UV-18, DASH-7, DASH-8 | A-6, A-10, AV-8, B-1, B-2, B-52, C-5, C-9 KC-10, C-17, C-130, C-135, C-137, C-141, E-3, E-4, E-6, R/F-4, F-5, F-14, F-15, F-16, F/A-18, F-22, FB-111, F-117, P-3, S-3, SR-71, T-1, T-2, T-37, T-38, T-39, T-42, T-45, TR-1, U-2 |

Table 1

Class A Runways are primarily intended for small light aircraft.  These runways do not have the potential or foreseeable requirement for development for use by high performance and large heavy aircraft.  Ordinarily, these runways are less than 8,000 feet long and have less than 10 percent of their operations that involve aircraft in the Class B category.  Class B Runways are primarily intended for high performance and large heavy aircraft.

Runway Lateral Clearance Zone:  A clear zone that is parallel and offset by 500’ from the centerline of the runway.  The runway lateral clearance zone’s lateral limits coincide with the limits of the primary surface.  The ends of the lateral clearance zone coincide with the runway ends.  The zone width is measured perpendicularly from the centerline of the runway and begins at the runway centerline.  The ground surface within this area must be clear of fixed or mobile objects and graded not to exceed 10% in either the longitudinal or transverse directions.  Fixed obstacles include manmade or natural features such as buildings (including air traffic control towers), trees, rocks, terrain, irregularities and any other features constituting possible hazards to moving aircraft. Navigational aids and meteorological equipment will be sited within these clearances where essential for their proper functioning to fulfill flight operation requirements. This area is to be clear of all obstacles except for the permissible deviations.  For Class B runways above ground drainage structures, including head walls are not permitted within 300’ of the runway edge.

Runway Clear Zones: Runway Clear Zones are areas on the ground, located at the ends of each runway.  They possess a high potential for accidents and their use is restricted to be compatible with aircraft operations.  Runway Clear Zones are required for the runway.  Table 2 shows the dimensional requirements for runway clear zones.

| Item No. | Item Description | Requirement | Remark |
| --- | --- | --- | --- |
| 1 | Length | 3000’ | Measured along the extended runway centerline beginning at the end of the primary surface. |
| 2 | Width | 3000’ | N/A |
| 3 | Longitudinal Grade | Max 10.0% | Graded area is to be cleared and grubbed of stumps and free of abrupt surface irregularities, ditches and ponding areas. No aboveground structures, objects, or roadways are permitted in the area to be graded, but gentle swales, subsurface drainage, covered culverts and underground structures are permissible. |
| 4 | Transverse Grade | Min 2.0% Max 10.0% | See Remark – Item 3 |

Table 2

Accident Potential Zones (APZ):  APZs are areas on the ground located beyond the clear zone of each runway.  They possess a potential for accidents and their use is restricted IAW DoD instruction no 4165.57.  Table 3 shows the dimensional requirements for runway APZs.

| Item No | Item Description | Requirement | Remarks |
| --- | --- | --- | --- |
| 1 | APZ 1 Length | 5,000’ | APZ 1 starts at the end of the Clear Zone and is centered and measured on the extended centerline extend. |
| 2 | APZ I Width | 3,000’ | N/A |
| 3 | APZ II Length | 7,000’ | APZ II starts at the end of APZ I, and is centered and measured on the runway centerline extend. |
| 4 | APZ II Width | 3,000’ | N/A |

Table 3

Airspace Imaginary Surfaces:  The area surrounding a runway that must be kept clear of objects that might damage an aircraft.  An object, either man-made or natural, which projects above an imaginary surface, is an obstruction.

Primary Surface: An imaginary surface symmetrically centered on the runway , extending 200’ beyond each runway end.  For Class B runways the width coincides with the lateral clearance zone width of 500’.  The elevation of any point on the primary surface is the same as the elevation of the nearest point on the runway centerline.

Approach-Departure Surface:  An inclined plane or combined inclined and horizontal planes arranged symmetrically about the runway centerline extended.  The first segment or the beginning of the inclined plane is coincident with the ends and edges of the primary surface, and the elevation of the centerline of the runway end.  This surface flares outward and upward from these points.

Inner Horizontal Surface:  An imaginary plane 150’ above the established airfield elevation.  The inner boundary intersects with the approach-departure clearance surface and the transitional surface.  The outer boundary is formed by scribing arcs with a radius of 7,500 feet from the centerline of each runway end and interconnecting those arcs with tangents.

Conical Surface: An imaginary surface that extends from the periphery of the inner horizontal surface outward and upward at a slope of 20 horizontal to one for a horizontal distance of 7,000’ to a height 500’, above the established airfield elevation. The conical surface connects the inner horizontal surface with the outer horizontal surface.

Outer Horizontal Surface:  An imaginary plane 500’ above the established airfield elevation extending outward from the outer periphery of the conical surface for a horizontal distance of 30,000 feet.

Transitional Surface: An imaginary surface that extends outward and upward at right angles to the runway centerline and the runway centerline extended at a slope ratio of 7H:1V. The transitional surface connects the primary and the approach departure clearance surfaces to the inner horizontal, the conical, and the outer horizontal surfaces.

Taxiway:  Taxiways provide for ground movement of fixed- and rotary-wing aircraft.  Taxiways connect the runways of the airfield with the parking and maintenance areas and provide access to hangars, docks, and various parking aprons or pads.  A wingtip clear zone distance is mandated based upon the size of the aircraft.  Table 4 shows the dimensional wingtip clearance requirements.

| Item No. | Item Description | Requirement |
| --- | --- | --- |
| 1 | Interior Taxilanes Wingspans<110’ | 20’ |
| 2 | Interior Taxilanes Wingspans>110’ | 30’ |
| 3 | Peripheral Taxilanes Wingspans <110’ | 30’ |
| 4 | Peripheral Taxilanes Wingspans >110’ | 50’ |

Table 4

Aircraft Parking Aprons:  Aircraft parking aprons are the paved areas required for aircraft parking, loading/unloading, and servicing.  They include the necessary maneuvering area for access and exit to parking positions.  Aprons will be designed to permit safe and controlled movement of aircraft under their own power.  A clearance from the apron boundary marking to fixed or mobile obstacles is mandated.  Table 5 shows the dimensional apron clearance requirements.

| Item No. | Item Description | Requirement |
| --- | --- | --- |
| 1 | Clearance from Apron Boundary Marking | 125’ |

Table 5

Exemption:  A type of waiver granted to fixed objects (facilities), which penetrate an imaginary surface or occupy a clear zone, but were constructed under previous standards.

Frangibility Requirement: All aboveground structures identified as permissible deviations, except as noted otherwise, must be constructed with breakable couplings or sections designed to support the desired load under specific local wind and ice conditions, but frangible enough to cause minimal damage to an aircraft if struck. Their shear resistance must be calculated with respect to local wind and ice load requirements, and other conditions of the installation. Two examples of such conditions are requirements for above grade aircraft arresting system components and mast-mounted electronic navigation or meteorological equipment whose performance would be adversely affected by movement caused by wind.

Permissible Deviations:  The following facilities do not require a waiver, if the facility is sited properly and meets the frangibility requirements.

Visual Air Navigational Facilities: This term identifies, as a type of facility, all lights, signs and other devices located on, and in the vicinity of, an airfield that provide a visual reference to pilots for guidance when operating aircraft in the air and on the ground. Commonly used visual air navigational facilities are:

Approach lights

Runway lighting systems

Taxiway lighting systems

Runway distance markers

Runway arresting system markers (locations of pendent cables).

Taxiway signs (pilots’ guidance, advisory and other informational signs on the airfield movement area)

Visual Glide Slope Indicator (VGSI) Systems

Radar Facilities:  These facilities provide air traffic controllers information on aircraft alignment, rate of descent, and relative position in the approach. These facilities include:

Airborne Radar Approach Reflectors

Precision Approach Radar (PAR) and PAR Reflectors

Mobile Ground Control Approach (GCA) Facility

Mobile Radar Approach Control (RAPCON) Facility

Remote Microwave Link Equipment

Emergency Generators, Maintenance and Personnel Facilities

Instrument Landing System (ILS):

ILS Localizer Antennas

Far Field Monitor (FFM)

ILS Glide Slope Antenna

Marker Beacons: Outer Marker, Middle Marker and Inner Marker beacons support instrument approach procedures.

Mobile Navigational Aids and Communication Facilities: Power generators for these facilities will be located as close to the equipment and in as small a site configuration as possible.

Mobile Air Traffic Control Towers (MATCT)

Terminal Very High Frequency Omnirange (TVOR) Facility and Very High Frequency Omnirange (VOR) Facility

Tactical Air Navigation (TACAN) Facility and Very High Frequency Omnidirectional Radio Range (VORTAC) Facility

Runway Supervisory Unit (RSU): An RSU is a transportable or permanent all-weather, control tower type facility used to control or monitor aircraft movement.

Transmissometer Facilities: Transmissometer facilities measure and record horizontal visibility. They are installed adjacent to the ILS runway. Measurements are in terms of runway visual range (RVR), a reference of how far a pilot of an aircraft should be able to see high intensity runway edge lights. A transmissometer installation consists of a projector, detector, and recording or readout unit.

Wind Measuring Set: The wind measuring set measures wind speed and direction. It consists of a transmitter, indicator and recorder. The transmitter is mounted on a mast and is sited where representative winds of the runway touchdown area can be measured. The recorder is installed in the weather observation building.

Temperature-Humidity Measuring Set: The temperature-humidity measuring set measures temperature and a dew point of free air passing over a sensor. The set consists of a transmitter and indicator.

Wind Direction Cones: Wind cone mountings are of three types. Type I is a hinged steel support; Type II is an anodized tapered aluminum hinged base support; and Type III is an “A” frame, fixed-base support with a pivoted center pipe support.

Landing Direction Indicator (Landing “T” or Tetrahedron)

Operational Facilities:

Aircraft Arresting Systems and Barriers (net engaging systems): A series of components used to engage an aircraft and absorb the forward momentum of a routine or emergency landing (or aborted take-off).

Warmup or Holding Pad:  The warmup or holding pad is a paved area adjacent to the taxiway and the runway end. It provides a means of bypassing aircraft being held at the runway end for various reasons.

Arm/Disarm Pad: Arm/disarm pads are used for arming aircraft just before takeoff and for disarming weapons retained or not expended upon the aircraft’s return.

Vehicle Control Signs and Traffic Lights: These signs and lights provide drivers with guidance on traffic routes, service yard areas, and similar places. They provide warning information at runway and taxiway crossings and other hazardous points. Vehicle control signs and traffic lights may be located on the airfield movement area (including apron) without a waiver to criteria.

Runway Distance Markers: These markers are required for runways used by jet aircraft and are recommended for runways used by propeller type aircraft.

Aircraft Security System: If a security system or fence is approved by the Air Force for alert apron security, such as the microwave fence sensor or similar system as required, approval of the siting by the MAJCOM operation and safety offices will constitute a permanent waiver to airfield criteria. No fence shall penetrate the Primary or Approach-Departure Clearance Surfaces.

Maintenance Facilities:

Jet Blast Deflectors: Jet blast deflectors are installed where continual jet engine run-up interferes with the parking or taxiing of aircraft, the movement of vehicles, the activities of maintenance personnel, or where it causes the erosion of pavement shoulders.

Floodlights: Floodlights illuminate aprons, alert stubs, specialized pads and other paved areas used for aircraft maintenance, loading/unloading, area security, and other reasons. Floodlights are exempt from apron clearance distance criteria.

Fire Hydrants: Fire hydrants may be installed within the apron clearance distances. Explosives Safety Barricades: When barricades are an element in an aircraft alert complex, they are exempt from apron clearance distance criteria.

Ground Support Equipment (Mobile): Examples of ground support equipment exempt under this category are: aerospace ground equipment, electrical carts, forklifts, towbar trailers, fire extinguisher carts, material handling equipment, flightline maintenance stands, and portable floodlights.

Flightline Vehicles: Flightline vehicles, such as pickup trucks and vans, are exempt from apron clearance criteria.

Ground Support Equipment (Stationary): Stationary ground support equipment and the associated safety and security components are exempt from apron clearance distance. Examples of exempt stationary ground support equipment are centralized aircraft support systems and pantograph refueling systems. This category also includes markers for petroleum, oils, and lubricants (POL) supply lines, communications and utility lines, and property demarcation.

Crew Chief Shack: This facility, sometimes identified as an Airfield Maintenance Unit, is a trailer or permanent prefabricated structure that may be located at the end of the runway, close to the arm/disarm pad of the apron edge. It may also be located on an area of the apron where it will not be an obstacle to taxiing aircraft.

Service Roads: Service roads may be located on the perimeter of alert aprons, around specialized aircraft parking pads, and similar apron areas.

Fencing and Barricades (Jersey Barriers): Fencing and barricades are erected on airfields for a variety of purposes. Guidance for locating fences and barricades is the same as guidance for locating service roads.  No fence shall penetrate the Primary or Approach-Departure Clearance Surfaces, or the graded area of the Clear Zone.

Wildlife Control Devices: Various devices such as propane cannons, sirens, and traps may require siting within the airfield environment for wildlife control.

Miscellaneous:

Telephone and Fire Alarm Systems

Trash Collection Containers

Landscaping Around Flightline Facilities

Other Apron Facilities: Examples of these type facilities are hangars, washracks, taxi-through alert shelters, air passenger terminals, movable passenger access platforms (jetways), and weather shelters for sentries.

Utility Access Points

Temporary Waivers:  Waivers granted by installation authorities that last for less than one year and are established for obstructions created by construction activities.

Permanent Waivers: Waivers granted by higher headquarters when one of the following criteria is met:

Natural geographical features violate criteria, and it is not economical or practical to remove them.

Existing facilities deviate from criteria but removal is not feasible.

Installation, construction, or erection of a required facility or equipment item according to criteria AFI 31-1132 is not practical.

Removal of the cause of the violation of criteria is not economical or practical.