Pacific Air Forces Implementation of Airfield Obstruction Management System Software

ESM 684 Final Project

By

Michal D. Holliday

1.0 INTRODUCTION	1

2.0 REQUIREMENT DEFINITION	2

2.1 Permanent Waivers & Waiver Authority	2

2.2 Waiver Tracking & Submittal Requirements, Installation Level	3

2.3 Waiver Tracking and Recording Requirements, MAJCOM Level	5

2.4 Facilities Not Requiring Waivers	6

3.0 AIRFIELD WAIVER BUSINESS PROCESS	7

3.1 Airfield Waiver Process Description	7

3.2 Data Inputs & Data Maintenance	9

4.0 PRIOR AIRFIELD OBSTRUCTION TRACKING METHODS	11

4.1 Pre-1998 Aircraft Accident Tracking Methods	11

4.2 Airfield Obstruction Reduction Program Development	11

4.4 Airfield Obstruction Reduction Program – Phase II	12

5.0 IDENTIFIED SOLUTION	17

5.1 GIS Defined	17

5.2 Airfield Waiver Business Process  - The GIS Perspective	18

5.3 GOTS vs. COTS	21

5.4 AOMS Development	21

5.5 PACAF Customization	23

5.6 Resource Allocation	26

6.0 FUTURE ENHANCEMENTS & CONCLUSION	28

## 1.0 INTRODUCTION

At approximately 0904 on the morning of 24 July 1998, an F-16, call sign Nomad 2, commenced takeoff on Misawa Air Base runway 10 as number 2 of a 4 ship BSA (basic surface attack) mission. The runway was being officially reported as “wet”, although it was not raining at the time. Takeoff of Nomad 1, the lead aircraft, was uneventful. The mishap pilot (MP) initiated a high-speed takeoff abort prior to landing gear rotation. The F-16’s hook engaged several lateral webbing strips of a “SAFE-BAR” webbed net barrier which was laying on the ground.  The pilot initiated ejection at a point corresponding with an aircraft inertial ground speed of approximately 56 knots. The ejection was successful; all egress systems functioned normally. The F-16 went off the paved overrun onto grass and struck concrete, metallic and wood elements of the approach lighting and ILS localizer antenna systems. The external fuel tanks and landing gear were heavily damaged, and post-crash fire ensued. The MP drifted into the fire prior to reaching the ground, and was badly burned, resulting in fatal injuries.

The approach lighting and ILS localizer antenna systems were obstructions that were suppose to be frangible.  Because they were not, a waiver had been granted, allowing these obstructions to remain within the active airfield.  Airfield obstructions are an Air Force-wide problem that are receiving Chief of Staff attention and require accurate locating, tracking, and monitoring to prevent further needless loss of life.  The purpose of this project was to work with a team of contractors, higher headquarters experts, and representatives from various unit-level organizations to implement a standard software solution that will allow active Air Force airfields to become safer for all entities that require their use.  This report will outline the airfield waiver requirements that must be fulfilled on an annual basis by both installation and MAJCOM level agencies.  Next, a historical review of waiver reporting will be presented, providing perspective to the advances brought by the identified solution.  Finally, the identified solution will be discussed, followed by outlooks for the future and concluding remarks.

## 2.0 REQUIREMENT DEFINITION

When obstructions violate airfield imaginary surfaces or safe clearance criteria as defined in Appendix 1 of this report, the obstruction must be analyzed to determine potential impact to aircraft operations.  This analysis is conducted in accordance with the criteria outlined in Air Force Manual 32-1132(I) Airfield and Heliport Planning and Design (the manual).  The following is a synopsis of the waiver process as outlined in the manual.

# 2.1 Permanent Waivers & Waiver Authority

Permanent waivers must be requested when any obstruction meets any of the following criteria:

A) Natural geographical features violate airfield criteria, and it is not economical or practical to remove the geographical feature.  For example, a cliff exists approximately 500’ from the end of the runway violating runway clear zone criteria, yet clearly not economical to remove.

B) Existing facilities deviate from airfield criteria but removal is not feasible.

C) Installation, construction, or erection of a required facility or equipment item according to criteria outlined in the manual is not practical.

D) Removal of the cause of the violation of criteria is not economical or practical.

Air Force Major Commands (MAJCOM) may waive deviations from airfield and airspace criteria in the manual. The responsible MAJCOM Civil Engineer approves the waiver after coordination with all appropriate staff offices and concurrence by the MAJCOM Directors of Operations and Safety.  This authority is not delegated below MAJCOM level within the Pacific Air Forces (PACAF) MAJCOM.

# 2.2 Waiver Tracking & Submittal Requirements, Installation Level

Waiver packages must be submitted annually to the MAJCOM authority for review and approval.  A typical waiver package consists of:

- A PACAF Form 48, Airfield/Airspace Violation Report, listing each airfield violation and providing pertinent information concerning said violation.  A sample PACAF Form 48 is shown as Figure 1.

- An E-1 map displaying the physical location of each airfield violation on the installation.  This is a standard map for any Air Force installation, depicting the installation layout, physical location and description of each obstruction.

Obstructions of a similar nature naturally grouped in a small area, such as trees, will be assigned only one waiver number, with the total number of items indicated in parentheses in the description column on the PACAF Form 48.  For example, if the waiver is for 26 trees, the description will read, “Trees (26 each).”  When documenting waiverable items, installation civil engineers typically group adjacent supporting items with a controlling obstruction, or group related items such as a series of drainage structures, as one waiver. For example, the base operations building violates the 7’ to 1’ transitional surface and apron clearance criteria. There are also four utility poles, a 36-inch tall fire hydrant, and numerous trees and shrubs located on the side of the building that is farthest away from the apron. These items are essential to provide architectural enhancement and utilities for this structure, but they also violate apron clearance criteria. Because these items are isolated from aircraft operations by the base operations building, they would not become a hazard to aircraft operations until the base operations building is relocated. Therefore, the base operations building is the controlling obstruction. The installation would document the base operations building as an exemption (constructed under previous standards) and develop one waiver request for all supporting structures to analyze impact to aircraft operations.

Figure 1 – PACAF FORM 48, Airfield/Airspace Violation Report

Development of the annual submittal of the PACAF Form 48 includes the following activities, conducted by the installation Base Civil Engineer or their staff:

- Coordinates with installation flight safety, airfield management, and flight operations offices to request confirmation of existing waivers or approval of new waivers.

- Annotates proposed waiver location on appropriate E series map for MAJCOM evaluation.  Airfield violations within the installation boundaries are annotated on the E-1 map, off-installation violations are annotated on the E-2 map.

- Develops a Military Construction Program or other project to systematically correct non-permanent waivers.

- Presents a summary of waived items to the Facility Board each year for information and action.

- Establishes a procedure for recording, reviewing, and acting on waivers.

- Requests a temporary waiver from the facility commander for any construction project that violates any airfield clearance criteria during or after the completion of the construction project. The base must request a temporary waiver at least 45 days before the scheduled construction start date, or an emergency temporary waiver when 45 days are not possible.

- Advises the MAJCOM of any canceled waivers.

# 2.3 Waiver Tracking and Recording Requirements, MAJCOM Level

In conjunction with the installation activities, the MAJCOM Civil Engineer or their staff must also perform the following activities:

- Coordinate with flight operations and flight safety offices to grant waivers.

- Set and enforce reasonable safety precautions.

- Monitor installation action to correct temporarily waived items within specified periods.

- Establish procedures to ensure an annual review of all waived items.

- Establish the administrative procedures for processing waivers.

- Maintain (for record) one copy of all pertinent documents relative to each waiver, including a record of staff coordination on actions at base and command levels.

# 2.4 Facilities Not Requiring Waivers

Facilities listed as permissible deviations in Appendix 1 of this report do not require waiver approval if sited properly.  Facilities constructed under previous standards should be documented as exemptions and programmed for replacement away from the airfield environment at the end of their normal life cycle, or when mission needs dictate earlier replacement.

To better understand the airfield waiver process, the day-to-day business process was researched, outlined and graphically depicted under a contract with Geo Insight International Inc (Geo Insight).

## 3.0 AIRFIELD WAIVER BUSINESS PROCESS

In October of 1999 Geo Insight International Inc. performed a Geospatial Information System, Business Process Needs Assessment.  As part of that assessment, the existing airfield waiver process was outlined and modeled.  The following summarized the airfield waiver business process as identified by Geo Insight and reviewed by PACAF Civil Engineering.

# 3.1 Airfield Waiver Process Description

The process begins when a request for an airfield waiver is received.  Information is collected about the project requiring an airfield waiver request and its intended site.  This can include contacting the requesting organization for more information, performing an onsite investigation, or examination of the airfield waiver regulations.  If it is determined that a waiver is not required, then it must be determined if the project requires classifications as a permissible deviation.  If it does not require such classification, then the requesting agency is notified that no waiver is required.  If it does require such classification or if it is determined that an airfield waiver is required, then a site plan must be drawn up.  This plan is typically generated using CADD software.  Once generated, the plan is forwarded in paper format to the appropriate organizations for approval.  If approved, the requesting organization is notified and the project is permanently added to drawings of the airfield and to a database of waiver items.  If the waiver is denied, then the customer is notified.  Typically, all project documents are stored in paper files and the CADD drawings are stored on a floppy disk in the project folder.  Figure 2 graphically depicts the airfield waiver process, as it currently exists.

Figure 2 Existing Airfield Business Process

# 3.2 Data Inputs & Data Maintenance

In the current process information is input as follows:

- Paper maps and drawings are accessed through the maintaining organization.  In most cases this is Civil Engineering, but can also include Communications, Supply, or Security Forces.

- Obstruction information is accessed through the maintaining organizations paper files.  This would include information such as size, function, or frangibility.

- Installation personnel from different organizations must perform an onsite inspection.  This team is typically composed of personnel from Civil Engineering, Airfield Management, and Flight Safety.

- The airfield waiver process owners must request information from other Civil Engineering organizations.  Typically this includes information such as construction methods, electrical requirements, or frangibility.

- The airfield waiver process owners within the Civil Engineering organization must request information from organizations outside of Civil Engineering.  Typically this includes requesting information from the airfield operations management organization or from the obstruction owner organization.

In the current process, information is maintained in the following formats:

- Paper maps are updated and maintained by the organization, little information is exchanged with other organizations.  Typically the maintaining organization the maintenance engineering flight within the Civil Engineer squadron, but other organizations such as Communications or Security Forces also update and maintain obstruction maps.

- Tabular data is stored in small spreadsheets on desktop hard drives or on floppy disks.  Rarely is this information available across the Internet, installation intranet, or installation network.

- Documents and forms are stored in paper files.  Additionally, all historical documents and forms are maintained in paper files, creating a legacy database only searchable by hand.

- Official regulations are stored in bound paper manuals.  Again, creating a system which can only be queried by hand.

In the current process distances, elevations, and areas are measured onsite with survey equipment.

Under the current process site plans are generated using CADD software.

Basically, the entire process is maintained using paper or flat files.  Data may be input via an electronic mode such as CADD or a database program, but these two electronic products are never electronically tied together.  The reduced capability to input, view, and analyze data is illustrated in section 4.0, a discussion of prior solutions.

## 4.0 PRIOR AIRFIELD OBSTRUCTION TRACKING METHODS

Prior to the accident outlined in section 1.0, airfield waivers were considered the answer to any airfield obstruction.  Anytime someone would desire to place a new object that violated airfield criteria, a waiver package was generated, submitted to the MAJCOM and approved.  Additionally, whenever an object appeared on the permissible deviation list, the object was automatically exempted without determining if the object had been sited properly or was frangible.

# 4.1 Pre-1998 Aircraft Accident Tracking Methods

The airfield waiver process before the 1998 aircraft accident was an archaic system that had remained essentially unchanged for over thrity years.  Each year the annual airfield violations tabular data package was generated entirely by hand.  At some point CADD, methods were introduced as a better method for generating the E-series maps, but there was never an electronic tie between these pieces of data.  Even the PACAF Form 48 was reproduced, and data was input to the form via typewriter.   Airfield obstructions were not considered a concern, and airfield waivers were generated whenever requested.  During this period, an accurate method did not exist to track remediation action, waivers corrected and moved to history, or obstructions meeting the criteria for permissible deviation status.  In fact, permissible deviations were not reported at all; therefore, installations were ignorant to the true number of airfield obstructions that existed in their airspace.

# 4.2 Airfield Obstruction Reduction Program Development

After the 1998 accident, an extensive review of the PACAF airfield waiver packages revealed that airfield waivers were not the solution, but the problem.  In a March 1999 letter from the PACAF Vice Commander to the PACAF installation commanders, it was identified that over 2,000 active obstruction waivers existed on PACAF airfields.  To compound matters, per the guidance outlined in section 2.0 of this paper, each airfield waiver could contain several obstructions of a similar or related nature.  Therefore, the identified 2,000 active waivers could actually represent as many as 10,000 or 15,000 obstructions.  In reality, the actual number of obstructions was unknown.

It was at this point that a joint effort between PACAF Civil Engineering, Flight Safety, and Airfield Management developed the Airfield Obstruction Reduction Program (AORP).   The premise of AORP is that the highest potential for an aircraft mishap exists within close proximity to the active runway(s) and that elimination efforts should focus on these obstructions, thus reducing the risk to in-flight aircraft.  Additionally, it was realized that several obstructions could be eliminated at little or no cost.  For example, frangible couplings for an airfield sign can be installed by in-house labor at little cost to the installation.  To capture both the airfield proximity and projected remediation cost, the AORP Matrix was developed and distributed to the field in May 1999.  The initial data received in October 1999 is shown in Figure 3.  By implementing the use of the AORP matrix, PACAF was finally able to capture quantitative information concerning obstructions affecting airfield operations.  This simple matrix was a simple, yet powerful tool used for funds advocating and eventually receipt of funds to enable the removal of several obstructions at Misawa AB, Japan.

Now data was captured concerning airfield obstructions and the cost to remove those of highest concern.  This presented PACAF with the ability to advocate for funding from Air Staff (Headquarters USAF) to remediate obstructions, a capability previously unrealized.  This new and unique method of categorizing airfield obstruction data was forwarded to higher headquarters for review and AF-wide implementation.

# 4.4 Airfield Obstruction Reduction Program – Phase II

Upon review of the PACAF AORP, Air Force experts concluded that the program, with a minor modification, was the proper method to categorize airfield obstruction data.  During the review process it was determined that obstruction removal should focus on correctable obstructions, near the runway surface.  Correctable obstructions are defined as those obstructions that are economically and operationally feasible to remove.  For example, the aircraft arresting system is economical to remove, but is necessary to mission operations.

Two new pieces of information emerged from the review of the AORP, the establishment of Priority Areas, and the development of a new obstruction-reporting database (AORP/DB).  The new priority areas were intuitive in nature, with the developing foundation being that aircraft are at highest risk by obstructions that are closest to the runway.  This was proved to be true by an Air Force Civil Engineering Support Agency (AFCESA) study, which proved that over 95% of all aircraft-obstruction accidents occur with a 500’ rectangle of the primary surface.  Therefore, these new priority areas focused on runway operations and are shown in Figure 4, while the AORP/DB sought to standardize the reporting procedures within the AF.

The development of AORP/DB presented several challenges, the majority of these resulting from the ambiguity within the reporting form and format.  The input form for AORP/DB is shown in Figure 5.    Information is limited and not required in a standard format.  Therefore, when inputting data in the “Distance From Centerline” field, responses included the following:

- 25’

- 25 feet

- 25-150 feet from centerline

The variety of answers prohibited querying the database for obstructions, thus limiting the actual value added by this product.  To effectively utilize this database, each data field for each record had to be checked to ensure that the data was in the appropriate format.  The man-hours and ultimately resources wasted on this data validation were immense.  It was at this point in the process that PACAF conceived the idea to tie this text and tabular data directly to the graphical map through the use of a Geographic Information System (GIS).

Figure 3 – Airfield Obstruction Reduction Program Matrix

Figure 4 – Primary Area Depiction as Defined by Air Force Expert Review

Figure 5 – Database Entry Screen for AORP Database

## 5.0 IDENTIFIED SOLUTION

# 5.1 GIS Defined

In the late twentieth century, demands for data depicting the topography and specific themes of the earth’s surface, such as natural resources, have accelerated greatly.  Stereo aerial photography and remotely sensed imagery have allowed photogrammetrists to map large areas with great accuracy.  The same technology also gave the geologists, soil scientists, ecologists, and land use specialists, enormous advantages for reconnaissance and semi-detailed mapping.  The resulting thematic maps were a source of useful information for resource exploitation and management.

The need for spatial data and spatial analyses is not just the domain of earth scientists.  Urban planners and cadastral agencies need detailed information about the distribution of land and resources in towns and cities.  Civil engineers need to plan the infrastructure routes, estimate construction costs, and determine mass-haul requirements.  The enormous infrastructure of what are collectively know as ‘utilities’ – that is water, gas, electricity, telephone lines, sewerage systems – all need to be recorded and manipulated as spatial data linked to maps.  Police departments need to know the spatial distribution of various types of crime, medical organizations and epidemiologist are interested in the distribution of sickness and disease, and commerce is interested in improving profitability through the optimization of the distribution of sales outlets and the identification of potential markets.  Today, there are many ways in which the political and economic processes responsible for resource management need to be assisted, enabling the best use of the available resources.

The first need is to learn what resources are available – how much fertile land, how much energy, and how many people?  The second focuses on the resources distributed; where they are, their extent, who owns them or and how can they be best utilized and managed.

The growing demands for more spatial data and better means to analyze that data can only be met by using computers.  The most critical new hardware element is the computer, which has conquered the workplace in most of the developed world.  Alongside the impressive development in computer hardware, the development of software has sparked a fundamental rethinking of the concepts underlying geographic information.  The separate specialties known as digital cartography, spatial analysis, remote sensing, multipurpose land information systems, and a host of others have converged to create an interdisciplinary focus on Geographic Information Systems (GIS).

There are dozens of definitions for the term GIS, each developed from a different perspective or disciplinary origin.  Some focus on the map connection, some stress the database or the software tool kit, and others emphasize applications such as decision support.  One of the most general definitions was developed by consensus among 30 specialists:

Geographic Information System – A system of hardware, software, data, people, organizations and institutional arrangements for collecting, storing, analyzing and disseminating information about areas of the earth.

# 5.2 Airfield Waiver Business Process  - The GIS Perspective

Part of the prior assessment conducted by Geo Insight was to determine the applicability of GIS within the airfield waiver process, and then model the revised business process under a GIS framework.  The following is a summary of the airfield waiver business process in a GIS framework, as identified by Geo Insight and reviewed by PACAF Civil Engineering.

With GIS, information collection becomes much quicker.  Information about the obstruction may be pulled from the GIS, reducing the need to contact the requesting agency for more information.  Onsite inspections could be eliminated due to the fact that most onsite inspections are performed to gather elevation data that is already maintained in the GIS.  Airfield waiver regulations could also be stored and searched electronically in the GIS.  Data analysis, which is generally limited to comparing distances and elevations to regulations, could be done quickly and accurately with the GIS.  Distribution and approval of site plans to other organizations could be done electronically and substantially more efficient through the GIS.  Once the waiver is approved it can be added to CADD drawings and inventoried in the GIS.  Obstruction documents could also be archived in the GIS.  Figure 6 depicts the airfield waiver process as it would be carried out when a GIS is in place.

With a GIS implementation, information would be input to the process through the use of the GIS.  All base maps, drawings and text and tabular information is accessed in the GIS.  Additionally, information would be maintained in the GIS.  Electronic maps and drawings can be maintained in concert with other Civil Engineering organizations, while all tabular data, documents, forms and manuals are stored in the GIS.  Distances, elevations and areas are measured quickly and accurately in the GIS.  Using this data, site plans would be generated with the GIS and/or CADD software.

Employing GIS in this process would provide several benefits.  The average labor hours required to perform the process for a single obstruction could be shortened by ten hours.  The majority of labor and time savings would also accrue from the elimination of field surveys.  A field survey requires several field team members to be on the airfield for several hours at a time. This is costly and also has the potential to disrupt airfield operations.  Such surveys could be eliminated with the use of a GIS.

Figure 6 – GIS Based Airfield Waiver Business Process

# 5.3 GOTS vs. COTS

Realizing that significant savings could be realized with the development of a GIS solution to the airfield waiver process, PACAF began researching available technologies that might meet the current need.  A consistent problem in the development of any software solution is the inability to export that solution to another agency/user and the inability for external programs to use the data collected by the software.  This is referred to as a closed-architecture solution and in the past, has been the common solution produced by government agencies.  Typically, the government manufactures propriety software, meeting the exact needs of the user, but also requiring government investment for any modification.  This is often referred to as a Government Off-The-Shelf (GOTS) solution.  A major goal in the final development of a GIS solution was the ability to allow not only other AF installations to use the program, but to also allow other programs to use the data collected by the program (i.e. open-architecture).   Additionally, PACAF desired to make use of commercially available software, often referred to as Commercial Off-The-Shelf (COTS) software, configurable to meet the users needs.  By using COTS software, the requirement for research and development of new COTS software is placed totally on the COTS software manufacturer.  Therefore, when Microsoft develops a new operating system, the COTS software company must carry the technological burden to create patches, new versions, etc. which allow their software to continue to operate in a new environment.  Additional design goals for this application included ease of use, minimal training requirements, and installation level maintenance.  Finally, the capability to generate the necessary reports, maps, and new waiver packages automatically was essential.

# 5.4 AOMS Development

Researching for an existing solution proved to be vital.  Since all active Air Force airfields have the same reporting requirements, it was likely that some agency had developed a solution that met many of these goals.  In fact, PACAF found that an existing Airfield Obstruction Management System (AOMS) solution met 80-90% of their goals and was being used by the United States Air Force Academy (AFA), various locations within the United States Air Forces Europe (USAFE), and various locations within the United States Air Combat Command (ACC).

These common goals have been achieved through the use of COTS software already widely in use throughout the Department of Defense (DoD) and many other government agencies.  An Environmental Science Research Incorporated (ESRI) product, ArcView ©, provides the graphic engine that allows obstructions to be viewed using existing map data with the added benefit of overlaying ortho-rectifed aerial photography.  This visual information is linked to a comprehensive Spatial Data Standard  (SDS) compliant Access © database that provides tabular information pertaining to the obstruction including its description, dimensions, imaginary surfaces/zones violated, and other critical data elements.  As a complete product, AOMS consists of:

- Twelve SDS 2.0 database tables

- Associated domain tables with values populated

- A database application using Access 2000

- An ArcView extension, currently using release 3.2

The ArcView 2-dimensional extension is installed and loaded into the ArcView environment in the same manner as other ESRI or third-party extensions.  Once loaded the program displays the basic airfield mapping data and provides functionality to perform simple database queries displaying the results graphically.  The extension will also perform other functions; including producing a 40” x 32 “ E-1 map, helping to initiate and coordinate a new waiver request, and printing reports.  Sample screens are included in Appendix 2.

Additional features include automated waiver generation/tracking and the capability to scan and maintain documents while associating them with a particular obstruction. A comprehensive user’s manual is also included with the application, and it provides step-by-step instructions for those individuals who maintain and update AOMS and for those who will simply be end users.

The database application will handle all linking of the SDS tables when obstruction data is entered into the form.  Appendix 3 contains graphics depicting the primary relationships between the twelve SDS tables used in AOMS and graphic data that will be supplied in maps and images.  Appendix 3 also presents each of the twelve SDS table used by AOMS.

# 5.5 PACAF Customization

As previously stated, AOMS met approximately 80 – 90% of the original software goals.  Upon review of the software the following inabilities were noted:

- AOMS did not incorporate the three Primary Areas as defined in section 4.0 of this paper.

- AOMS could not generate a PACAF Form 48 Airfield/Airspace Violation report.

- AOMS did not allow custom queries to be developed and performed on the existing database.

Although seemingly minor in nature, each of these items were key to producing an automated solution that was usable by field and headquarters personnel.  Therefore, in November 2000, PACAF contracted to provide a revised and populated AOMS configuration at all nine PACAF installations.  The statement of work, cost estimate, installation location map and contract schedule, are included at Appendix 4.

The customization of AOMS began with a statement of work (SOW) development meeting.  This meeting was held at Hickam AFB HI and attended by representatives from Higgenbotham/Briggs & Associates (HB&A) , Geo Insight, and PACAF Civil Engineering.  The purpose of this meeting was to determine the current capabilities of AOMS, outline the enhancements desired by PACAF, and develop a rough order of magnitude (ROM) cost estimate.  Under the guidance of the project manager, a SOW was developed that defined the known additional capabilities desired, but also allowed for additional capabilities to be defined by a group of functional experts.  Additionally it was decided Hickam AFB would serve as a pilot project, outlining all new functionality, data fields to gather, or reports that should be generated.  The final product developed for Hickam would then be implemented at each of the eight remaining PACAF installations, with the only change being the foundation data.  At the end of the two-day planning session the project manager felt confident that the SOW adequately defined the required work and the contractor provided a budgetary ROM.  A subsequent contract was let through GSA to Geo Insight as the prime contractor and HB&A as the sub-contractor.

The formal contract began with a kickoff meeting between the government and contractor was held at Hickam AFB HI on 28 Nov 2000.  Present at the kickoff meeting were representatives from the following installation level and MAJCOM level organizations:

- Airfield Management

- Flight Safety

- Civil Engineering

During the kickoff meeting the existing capabilities of the program were displayed and new improvements were discussed.  At this time no additional improvements were requested and the contractor began collection of obstruction data for Hickam AFB.  With Hickam being the pilot project, the project manager requested three submittals prior to the final installation.  The contractor agreed to provide a 35% submittal outlining the database schema and data fields; a 65% showing initial Hickam datasets and program functionality; and a 95% draft final in which all program elements should be functional and the data should be robust and viable.

Per the contract schedule, the 35% and 65% arrived for government review.  During the 65% review several comments were made concerning desire for increased capability.  Although minor in nature, several of these items had great impact on the end-user and their ability to use the program.  The entire list of comments is included at Appendix 5.  Several negotiations were required with the prime and sub-contractors to persuade these parties to incorporate the majority of the comments.  Some of the comments/requirements requested extensive programming that was not included in the SOW, nor did the comment add significant capability to the program, therefore some of the user comments were not incorporated into the 95% deliverable.

Due to key installation personnel being deployed for an extended period of time, the 65% review meeting was postponed.  It was determined by the project manager that the deployed personnel’s attendance at the 65% review meeting was vital to project success and that this delay would not significantly affect the implementation schedules at the remaining eight PACAF installations.  The subsequent delay resulted in the 95% submittal not being delivered until 17 April 2001.  The 95% meets all the goals and objectives as outlined in the SOW and was approved for final delivery. Incompatible schedules require that the 100% installation and subsequent training take place the week of 21 May 2001.  While the implementation schedule was completely outlined at the contract start, unknown conflicts have required changes to the implementation schedule.  The remaining installations are resolving exercise schedule, weather, and other factors with the program manager to ensure that AOMS will be operational at all PACAF installations by 31 December 2001.

The extent of software capabilities will be displayed during the oral presentation.

# 5.6 Resource Allocation

Effective execution of this project required the allocation of two vital resources; manpower and funding.  Significant logistical requirements must be coordinated at each installation.  As a minimum each installation is required to provide the contractor with installation drawings in an electronic format, a copy of the most recent waiver package (PACAF Form 48), and airfield escorts during the airfield survey.  Although the project manager at PACAF accomplished significant project organization, the physical divergence of the PACAF installations required the identification of an installation point-of-contact.  This POC would be the primary liaison between the government and HB&A, coordinating all contractor activities at the given location.  The appointment of this POC was accomplished via an official memo sent from the MAJCOM Civil Engineer to each installation wing commander.  Interfacing at this level allowed the program implementation to receive visibility from senior leadership and ensured the implementation would flow smoothly.

Shortly after the project kick-off meeting the MAJCOM project manager was promoted to a supervisory position over the GIS Implementation Section.  This promotion laid more responsibility upon the project manager, but also provided additional resources for the project manager’s use.  Therefore the project manager delegated the implementation at each installation to a GIS professional, now under his supervision.  Since this project is a GIS development, the assignment of this implementation to the project manager’s subordinates provided the GIS professionals an opportunity to interface with a new GIS program, installation POC’s working GIS issues, and a third-party contractor providing GIS solutions. Delegating this work allowed the project manager to focus on the greater perspective of GIS implementation across the PACAF installations, while retaining adequate control ensuring the project goals are accomplished.

Securing adequate funding for project execution was relatively simple in comparison to the bureaucratic difficulties required to acquire funds for other requirements.  MAJCOM level senior-leadership was very cognizant of the attention airfield obstructions were receiving and desired to produce a modern solution to this problem.  When AOMS was showcased at a series of commanders conferences, the question was not how much implementation would cost, but when AOMS would be installed at PACAF installations.  With this support, funding was immediately produced for implementation, with additional funds available should contingencies arise.   This was a major accomplishment as this funding not only covered the development of the software, but funding was also secured to fund contractor accomplishment of the airfield surveys, on-site software training, and final installation.  The initial position of the project manager had been that development, training, and installation had to be fully funded if this program was to be a success, and due to proper program salesmanship this position was supported by senior leadership, who allocated necessary funds to accomplish all project phases at each installation.  Thus far the project has been executed, as planned and additional funds have not been required.

## 6.0 FUTURE ENHANCEMENTS & CONCLUSION

As program awareness and capability increases, future enhancements continue to be identified.  Future enhancements provide capabilities which will increase planning potential and communication between installation, MAJCOM and Air Staff.

An amendment to current policy allowing recognition of a “predominant building line” is being contemplated for inclusion in AFI 32-1132.   This amendment would allow installations with a significant number of facility obstructions, located along a common imaginary line, to be exempt from requiring waiver approval or removal.  With the capabilities of a GIS-based obstruction-tracking program, the development of this capability is very simple, but very beneficial to Air Force installations.  Simply put, all measurements and necessary adjustments to airfield waivers can be conducted at a single point of input and eliminate unnecessary paperwork throughout the installation.

An additional planning capability being built is the ability to perform “what if” type analysis.  AOMS does not currently answer the question of which imaginary surfaces are violated by the placement of an obstruction.  This capability, although realized as being very valuable, was outside the scope of work for this contract.  Additionally, it appears that other Air Force entities are funding this development and PACAF may be able to acquire this capability at no cost.

The greatest expansion of this software will be the developing AOMS into an Internet based application, allowing users at all levels access to the most recent data, simplification of the annual submittal process, and reduction of routine work to a new level.  This expansion is being researched under the current contract.  Cost will determine when this capability is realized.

AOMS has changed the entire obstruction reporting process within PACAF.  Where paper and pencil were used to maintain a less than accurate record; digital media now presents a precise and current relational database.  Where paper reports of various format and quality once required massive analysis, standard reports and accurate maps allow for accelerated evaluation and approval.  PACAF is leading the way in the Air Force with the execution of AOMS across the command.  Other Air Force MAJCOM’s are attempting full-scale efforts and it is expected that AOMS will become the Air Force defacto software standard in the near future.

Simply implementing AOMS will not prevent another Misawa-type accident; but not implementing a management tool will guarantee another accident.   Airfield managers, flight safety personnel, and civil engineers must know what the hazards are if they are to be properly constructed or better yet, removed.

BIBLIOGRAPHY

Ates,M. 1997 Air Force Manual 32-1132 (I) Airfield and Heliport Planning and Design. Air Force Civil Engineer Support Agency, United States Air Force

Burrough, P.A. and McDonnell, R.A. 1998 Principles of Geographical Information Systems.  Oxford University Press Inc.

Chrisman, N.R. 1997 Exploring Geographic Information Systems.  John Wiley & Sons Inc.

Clarke, K.C. 1997 Getting Started with Geographic Information Systems.  Prentice-Hall Inc.

McCann F. and Bedard L. Airfield Obstruction Management System.  Higginbotham/Briggs & Associates.

Riegelmann E. Geospatial Information System Implementation Plan, Volume II – Business Process Needs Assessment. Geo InSight International Inc.